| From | Brinkhof (Rien Broekstra and Maarten Groeneveld) |
| --- | --- |
| To | Tech Sonic (Byron Kieser) |
| Copy to |  |
| Date | 19 December 2025 |
| Reference | RB//2569019 |
| Re | Infringement analysis of KOKS Ecosonic ultrasonic baths vs. Tech Sonic EP 2 516 074 B1 |

# Executive summary

The offering and sale by KOKS of the KOKS Ecosonic 3, Ecosonic 7 and Ecosonic 9 (the “Devices”) for use in countries where EP 2 516 074 (“EP 074”) is in force likely constitutes at least indirect infringement of multiple claims of Tech Sonic’s EP 074 Patent. Furthermore, the actual use by KOKS of the Devices for demonstration purposes likely directly infringes the Dutch part of EP 074. Accordingly, Tech Sonic is likely entitled to injunctive relief, recall and damages and/or profit surrender.

It less likely that Tech Sonic can stop Koks’ manufacture of the Devices. Independent product claim 1 requires “a system comprising an apparatus … and a cleaning liquid”. Koks may successfully argue that the manufacture of the apparatus without also providing/inserting the cleaning liquid does not infringe this claim. Also, Koks may remain entitled to offer, sell and deliver the Devices insofar they will be used outside of the Netherlands, or – if the 074 Patent is asserted in a Dutch cross-border case or at the UPC – outside of the territory of the EPC or UPC respectively.

An allegation of infringement of the European Patent may exert significant pressure on Koks. Since at least indirect infringement is likely, we think to have more than sufficient basis for sending out a warning letter. To increase pressure, such a warning letter may also mention that Koks’ customers are potential targets for follow-up action.

Should the warning letter ultimately be followed up by litigation, Tech Sonic should consider to assert international, patents from the same family as EP 074 alongside EP 074 itself against Koks in one single Dutch cross-border proceeding. The Dutch court is likely to accept jurisdiction against Dutch-domiciled Koks entities as well as their foreign group companies and distributors to hear infringement claims of patents worldwide.

# The 074 Patent

## Bibliographic data

EP 2 516 074 is entitled “Apparatus for cleaning industrial components”. It was granted based on PCT patent application WO 2011/075831 A2. The PCT application date is 22/12/2010. It invokes the priority of US 289050P with a priority date of 22/12/2009.

According to the national registers, EP 074 is validated and in force in Austria (AT), Belgium (BE), Bulgaria (BG), Czech Republic (CZ), Germany (DE), Denmark (DK), Spain (ES), Finland (FI) France (FR) Greece (GR), Croatia (HR), Hungary (HU), Ireland (IE), Italy (IT), Lithuania (LT) the Netherlands (NL), Norway (NO), Poland (PL), Portugal (PT), Romania (RO), Serbia (RS), Sweden (SV), Slovakia (SK) and the United Kingdom (GB). Tech Sonic Limited Partnership is the registered proprietor.

Parallel patents appear in force in the United States, Japan, China, Brazil, Canada, Mexico, The Philippines, Russia, Singapore, South-Africa and South Korea.

## Prosecution history

From the prosecution file it is apparent that after several rounds of claim amendments, the applicant on 10 August 2019 filed a request combining original claims 1, 6 and 9 into a proposed new claim 1.

During oral proceedings, proposed new claim 1 was considered unclear because (a) the spacing feature is unclear because it depends on the cleaning fluid, which is not present in the claims; and (b) the generation of a particular power density is actually a method feature, not an apparatus feature.

With respect to objection (b), the applicant successfully argued that this feature is the result of the positioning of the transducers in the container.

To overcome objection (a), the applicant included the features of original claim 23 into claim 1, defining the cleaning liquid. This was considered clear, novel and inventive over the prior art on file. After some editorial corrections, the patent was then granted.

Current claim 1 is a product claim which reads:

| 1A | A system comprising an apparatus for cleaning industrial components and a cleaning liquid, comprising: |
| --- | --- |
| 1B | a liquid container (200) having sidewalls (202, 203; 309, 310; 403, 404, 405, 406) defining a liquid enclosure for containing the cleaning liquid, |
| 1C | the liquid container (200) having a component-receiving area spaced from the sidewalls (202, 203; 309, 310; 403, 404, 405, 406); and |
| 1D | ultrasonic transducers (207; 315; 409; 700; 912) having an operating frequency and a wavelength in the cleaning liquid and secured to at least a portion of the liquid container (200) at a spacing of between 2 and 10 wavelengths in the cleaning liquid, |
| 1E | wherein in operation the ultrasonic transducers (207; 315; 409; 700; 912) generate a power density in the component-receiving area of the liquid container (200) that is greater than an average power density of the liquid container, wherein the ultrasonic transducers (207; 315; 409; 700; 912) are resonating rod transducers secured to the inner surface of the liquid container (200); and |
| 1F | wherein the liquid container (200) comprises an aqueous cleaning solution comprising at least one of solvent additives, an acid solution and an alkaline solution. |

# Infringement analysis

Under Dutch law, a patent is infringed if a third party:

- performs a reserved act (manufacture, offer, bring to market, use, or import and/or stocking product for those purposes), with a product that fulfils all features of at least one claim of the patent, and/or

- uses a method that fulfils all features of at least one claim of the patent; and/or

- performs a reserved act with a product that has been directly obtained by a claimed method; and/or

- offers or delivers an essential element of the invention, if the third party knows or ought to know the element is suitable and intended for bringing the claimed invention into effect in the territory of the Netherlands (indirect infringement).

Our analysis is based on the following public sources:

- The website https://koks.com/, in particular:

  - https://koks.com/nl/product/ultrasoonbaden

  - https://koks.com/nl/ultrasone-reinigingssystemen

  - https://koks.com/nl/product/koks-ecosonic-3-meter-bad

  - https://koks.com/nl/product/koks-ecosonic-7-meter-bad

  - https://koks.com/nl/product/koks-ecosonic-9-meter-bad

  - https://koks.com/Files/Images/Corporate%20presentatie/Presentatie%20KOKS%20Group_Corporate_%20NEDERLANDS%20-%20DEFINITIEF%20-%20Nieuwe%20huisstijl.pdf

- Video’s linked from the koks.com website, hosted on youtube:

  - https://www.youtube.com/watch?v=TmsFWg-bnAU (the “Animation Video”)

  - https://www.youtube.com/watch?v=fmmJ1hm0O-Q (the “Demonstration video”

  - (https://www.youtube.com/watch?v=LRHom8KeFkY (the “9m Animation Video”)

- Information from the Dutch Chamber of Commerce

## Fulfillment of claim 1 by KOKS Ecosonic 3m

We believe it is likely that the KOKS Ecosonic 3 fulfills all features of claims 1-6, with the caveat that the bath is not offered/sold including a particular cleaning liquid.

We further believe it is likely that the use of a KOKS Ecosonic 3 by a customer of KOKS fulfills all features of method claims 8, 9, 11, 12 and 13, so that the offering and sale of an Ecosonic 3 that is to be used in the territory of the Netherlands indirectly infringes these claims. Similar infringement theories are applicable if foreign parts of the European Patent are asserted.

A mapping of the claim features to features of the KOKS Ecosonic 3m is shown below:

| Claim 1 |  |
| --- | --- |
| 1A | A system comprising an apparatus for cleaning industrial components and a cleaning liquid, comprising The Ecosonic 3m is an apparatus for cleaning industrial components: KOKS’s website (https://koks.com/nl/product/ultrasoonbaden) asserts In English: Why are ultrasonic baths essential for industrial cleaning? Our ultrasonic baths offer unparalleled depth of cleaning. They can clean delicate components without mechanical damage, resulting in an extended equipment lifespan. Furthermore, they reduce the need for manual scrubbing and the use of chemical cleaning agents, resulting in more efficient and environmentally friendly processes. What cleaning tasks are ultrasonic baths suitable for? Ultrasonic baths can be used to clean heat exchangers, filters, and heavily contaminated objects, among other things. They can deeply clean the internal and external surfaces of these components, improving efficiency and extending the lifespan of the machinery. In short, KOKS ultrasonic baths are an indispensable addition to industrial cleaning processes. The Ecosonic 3m’s only use is with a cleaning liquid: KOKS’ website further asserts that the device is used with a cleaning liquid: How does an ultrasonic bath work? Objects are placed in a tank containing a cleaning fluid. Ultrasonic waves are passed through the fluid, creating cavitation. This process creates tiny air bubbles that explode upon contact with the objects, dislodging dirt and contaminants. Koks is showing a video on its website with the Ecosonic 3m in actual use. That video shows that KOKS uses a system according to feature 1A, which can support direct infringement of claim 1. We note that it is not apparent that KOKS offers or sells the actual cleaning fluids along with their ultrasonic baths. While the use of the ultrasonic bath by Koks themselves (as in the Demonstration video) fulfils feature 1A, the mere offering or sale of an ultrasonic bath arguably does not directly infringe claim 1. We expect no difficulties, however, establishing that offering or sale of KOKS baths indirectly infringe feature 1A, as it is abundantly clear that the bath must be used with a cleaning liquid. Feature 1A is fulfilled, potentially through indirect infringement. |
| 1B | a liquid container (200) having sidewalls (202, 203; 309, 310; 403, 404, 405, 406) defining a liquid enclosure for containing the cleaning liquid A screenshot from the Animation Video (00:48) clearly shows a liquid container having sidewalls defining a liquid enclosure for containing the cleaning liquid Feature 1B is fulfilled. |
| 1C | the liquid container (200) having a component-receiving area spaced from the sidewalls (202, 203; 309, 310; 403, 404, 405, 406); and The Animation Video shows that the Ecosonic 3 comprises a separate basket intended to receive components to be cleaned. There is space on all sides between the baskets and the sidewalls: The Demonstration Video shows explicitly that the basket is intended to hold components: Feature 1C is fulfilled. |
| 1D | ultrasonic transducers (207; 315; 409; 700; 912) having an operating fre-quency and a wavelength in the cleaning liquid and secured to at least a portion of the liquid container (200) at a spacing of between 2 and 10 wave-lengths in the cleaning liquid, According to the Demonstration Video, “the vessel comes with 20 transducers, 2.000 watts each”. The Animation Video and pictures on the website (The Koks website (https://koks.com/nl/ultrasone-reinigingssystemen) show the transducers being secured to the liquid container at their top end, The Ecosonic 3m thus has ultrasonic transducers secured to at least a portion of the liquid container. The specifications on the Koks website (https://koks.com/nl/product/koks-ecosonic-3-meter-bad) show that the transducers operate at a frequency of 25 KHz: The wavelength of a wave is its speed through a medium divided by its frequency (λ=v/f). The speed of sound in water varies between approximately 1552 m/s at 60C to 1555 m/s at 80C. The corresponding wavelength thus is approximately 6,2cm. 2-10 wavelengths then corresponds to 12,4-62 cm. This is broadly in alignment with the patent (para. [0019]) whereby we note that the metric values in that paragraph appear to be conversions from relatively coarse imperial values. In the Ecosonic 3m, there are ten transducers on each side of the wall, approximately evenly spaced: This puts their spacing at approximately 25-30 cm, i.e. approximately 4,0-4,8 wavelengths. This is right in the middle of the claimed range. Feature 1D is therefore fulfilled. |
| 1E | wherein in operation the ultrasonic transducers (207; 315; 409; 700; 912) generate a power density in the component-receiving area of the liquid container (200) that is greater than an average power density of the liquid container, wherein the ultrasonic transducers (207; 315; 409; 700; 912) are resonating rod transducers secured to the inner surface of the liquid container (200); and From the images above, it is clear that the transducers in the Ecosonic 3m are rod transducers, and that they are secured to the inner surface of the liquid container. The Patent explains that placement of the rods 2-10 wavelengths apart results in interference. That would entail local maxima and minima within the fluids. There are thus necessarily certain locations within the component-receiving area where the maximum power density is greater than the average power density within the entire container. We further note that Koks seems to exploit this feature. The demonstration video describes: “we can program it that it [the basked comprising the components, RB] will continuously go up and down by 8 centimeters. [This] will increase the cleaning process of the components inside the basket because the ultrasonic wave and the effect of the ultrasonic wave will be each time on a different spot of the to be cleaned equipment inside the basket” This indicates that the power density is not completely uniform within the liquid, but that the placement of the component inside the liquid matters. Note: we are sufficiently confident on fulfilment of this feature to issue a warning letter. Should it however come to litigation, we would likely require some kind of further evidence which not only shows that there is interference between the rods, but also that that interference leads to positions inside the basket that where the power dentisty exceed the average. This could be done, for example, through mathematical simulation. Feature 1E is fulfilled. |
| 1F | wherein the liquid container (200) comprises an aqueous cleaning solution comprising at least one of solvent additives, an acid solution and an alkaline solution While this claim feature is broad, covering any non-ph neutral aeqeous solution, plus a ph-neutral solution with some solvent additive, we have not found clear evidence of the composition of the cleaning liquids to be used in the KOKS bath The Demonstration Video asserts that “With this vessel, we re-use the chemical fluid for the cleaning process”. The cleaning fluid can be assumed to be aqueous, as there are “(Waste) water savings”). It can reasonably be assumed that any aqueous chemical fluid is either acid, alkaline, or contains a solvent. Furthermore, the Koks website states: In English: Where can I buy an ultrasonic bath? KOKS is a trusted manufacturer and supplier of high-quality industrial solutions, including ultrasonic baths designed to provide thorough and efficient cleaning. To purchase an ultrasonic bath, please contact us directly. Our experts will provide you with comprehensive information about the various ultrasonic baths, their technical specifications, and prices. We recommend discussing your specific cleaning needs with us so you can choose the right ultrasonic bath for your industrial requirements. Assuming that particular industrial requirements necessitate the use of alkalines, acids, and/or solvents, Koks will thus knowingly sell a bath to a customer with such a requirements. Nevertheless, it would be useful to adduce further evidence that any cleaning liquid that will be used with the Ecosonic devices, falls within scope of this claim. If we can show that, the same considerations re: indirect infringement apply as with feature 1A Feature 1F: further evidence preferred – but likely fulfilled, possibly through indirect infringement. |
| Claim 2 |  |
| 2 | … wherein the transducers (207; 315; 409; 700; 912) resonate between 20 kHz and 30 kHz, with a typical centre frequency of 25 kHz The specifications on the Koks website (https://koks.com/nl/product/koks-ecosonic-3-meter-bad) show that the transducers operate at a frequency of 25 KHz: Claim 2 is fulfilled |
| Claim 3 |  |
| 3 | wherein the container (200) is sufficiently large to receive a set of heat exchanger tubes (106) between 0,6 m and 45,7 m in length and between 0,15 m and 3,7 m in diameter According to the specifications on the KOKS website (https://koks.com/nl/product/koks-ecosonic-3-meter-bad), the basket size is 3,1 x 1,0 x 1,0 m. This allows receiving g a heat exchanger tube within the ranges of claim 3. Claim 3 is fulfilled |
| Claim 4 |  |
| 4 | wherein the liquid container (200) comprises a bottom surface (201) that is one of sloped, flat, concave or "V" shaped. Depending on viewpoint, the bottom surface is all of sloped, concave and v-shaped. We believe this is sufficient for fulfilment of claim 4, although an argument could be made that the claim requires that the bottom surface has only one of these shapes. Claim 4 is likely fulfilled |
| Claim 5 |  |
| 5 | wherein the transducers (207) generate a power density within the liquid container (200) when filled with liquid of between 2,64 - 15,85 Watts/liter According to the Demonstration Video: “The volume of the vessel is 5500 liters which will create with each transducer in it 9 watts [per] liter”. According to the website specification, the power density is approximately 7,6 Watt per liter: Both fall squarely within the range of claim 5. Claim 5 is fulfilled |
| Claim 6 |  |
| 6 | wherein the transducers (207; 315; 409; 700; 912) are mounted vertically to the inner surface of the liquid container (200) and are mounted using a compliant clamping at a top of the transducer (207; 315; 409; 700; 912), and a mount device that does not restrict motion along the axis of the resonant rod. A screenshot from the Demonstration video shows that the transducers are mounted vertically, at their top end only. It is reasonably likely that they are mounted using a clamp. We believe this is sufficient to assert infringement of claim 6 – although it might turn out the top end is mounted in a different manner Claim 6 is likely fulfilled |
| Claim 8 |  |
| 8A | A method of cleaning industrial components, comprising the steps of: The Ecosonic 3 is clearly used in a method for cleaning industrial components. Feature 8A is fulfilled |
| 8B | securing resonating rod ultrasonic transducers (207; 315; 409; 700; 912) to an inner surface of sidewalls (202, 203; 309, 310; 403, 404, 405, 406) of at least a portion of a liquid container (200) at a spacing of between 2 and 10 wavelengths in the cleaning liquid based on the operating frequency and the operating wavelength of the ultrasonic transducers (207; 315; 409; 700; 912) in the cleaning liquid; The Ecosonic 3 has resonating rod ultrasonic transducers secured at a spacing of approximately 4,0-4,8 wavelengths based on an operating frequency of 25KHz and an operating wavelength of 6,2cm (see above claim feature 1D. Feature 8B is fulfilled |
| 8C | introducing the cleaning liquid into the liquid container (200) such that a minimum liquid level is reached and all ultrasonic transducers are submerged in the cleaning liquid; The Animation Video and the Demonstration Video show that, in use, cleaning liquid is introduced in the liquid container such the all ultrasonic transducers are submerged in the cleaning liquid: Feature 8C is fulfilled |
| 8D | introducing an industrial component into the cleaning liquid and positioning the industrial component in a component-receiving area of the liquid container (200) that is spaced from a sidewall (202, 203, 204, 205) of the liquid container (200); and This is fulfilled, see Feature 1C above Feature 8D is fulfilled |
| 8E | operating the ultrasonic transducers (207; 315; 409; 700; 912) to generate a power density in the component-receiving area of the liquid container (200) that is greater than an average power density in the liquid container (200) This is fulfilled, see Feature 1E |
| Claims 9, 11, 12, 13 |  |
|  | These are fulfilled for the same reasons as claims 2, 4, 5, 6. |

## Koks Ecosonic 7

The Koks Ecosonic 7 is a similar device as the Ecosonic 3, featuring a larger vessel. It fulfils the features of the claims for the same reason as the Ecosonic 3, with the following remarks/differences:

- Feature 1D/8B: According to the specifications (https://koks.com/nl/product/koks-ecosonic-7-meter-bad), the Ecosonic 7 features 56 transducers, 28 on each side, approximately evenly spaced. As with the Ecosonic 3, they are rod transducers operating at 25 kHz, 2kW each

- Note that the picture on the website shows more transducers – likely this picture is of a larger model.

- The bath has a usable length of approximately 7 meters. This brings the spacing to about approximately 25 cm or four wavelengths. This is within the range of feature 1D.

- Claim 3: the bath is 7 x 1,8m, which is sufficiently large to receive a set of heat exchanger tubes in the range of claim 3.

- Claim 4, 11: The bottom surface is concave, sloped and v-shaped. See the image above.

- Claim 5, 12: the power density is specified to be 5,4 Watt per liter

## Koks Ecosonic 9

The Koks Ecosonic 9 is a similar device as the Ecosonic 3 and 7,  but with an even larger vessel. It fulfils the features of the claims for the same reason as the Ecosonic 3 and 7, with the following remarks/differences:

- According to the specifications (https://koks.com/nl/product/koks-ecosonic-9-meter-bad) the Ecosonic 9 features 80 transducers, 40 on each side, approximately evenly spaced. As with the Ecosonic 3, they are rod transducers operating at 25 kHz, 2kW each

- The 9m animation video clearly shows the evenly spaced transducers:

- The bath has a usable length of approximately 9 meters. This brings the spacing to about approximately 22,5 cm or 3,6 wavelengths. This is within the range of feature 1D.

- Claim 3: the bath is 9 x 2,1m, which is sufficiently large to receive a set of heat exchanger tubes in the range of claim 3

- Claim 4, 11: We have no clear image of the bottom surface of the 9 meter model, although the 9m Animation Video suggests that it is at least concave and v-shaped:

- Claim 5, 12: the power density is specified to be 5,4 Watt per liter

## Offering of Koks Ecosonic baths

On its website www.koks.com, KOKS offers various ultrasonic bath models for rent and for sale. In particular, the KOKS Ecosonic 3 meter, 7 meter and 9 meter baths. See, e.g. https://koks.com/nl/ultrasone-reinigingssystemen/.

The website offers these devices worldwide. Koks also specifically targets customers in at least in the Netherlands, the United Kingdom, Germany, France, Spain, and Czech Republic, and the United States, as their website can be set to the respective languages.

Furthermore, Koks refers to dealers in, inter alia, the Netherlands, Norway, Czech Republic, Finland, Poland, the UK, Slovak Republic, Spain and Germany (https://koks.com/nl/dealer-locator).

The website allows to request a quote for specifically the 3m, 7m and 9m ecosonic devices. See, https://koks.com/nl/offerte-algemeen?ProdID=PROD23&VarID=  https://koks.com/nl/offerte-algemeen?ProdID=PROD70&VarID= and https://koks.com/nl/offerte-algemeen?ProdID=PROD71&VarID=

The above clearly established the offering of all three models of the Ecosonic.

Since the baths exhibit all features except for the cleaning liquid, the baths are clearly means essential to the invention. Further, in offering these baths to the Netherlands, Koks must be aware that the baths are suitable and intended for putting the invention of the aforementioned claims into effect. This offering therefore infringes the Dutch part of the Patent, and likely also all other validated parts of the European Patent.

## Use of Koks Ecosonic baths

The Demonstration Video shows that Koks manufactured a complete system according to claim 1 and its dependent claims, as well as its use according to claim 8 and its independent claims. The video was posted in January 2025. We also note that the first occurrence of the Ecosonic product line on the koks.com website took place mid 2020. This makes it highly likely that the use took place after the grant of EP 074 in January 2020. This use therefore infringed the Dutch part of the Patent.

# KOKS’ financial information and exposure

Koks is a medium-sized, family owned enterprise with a yearly group turnover of approximately EUR 70-80 million and a balance sheet total of EUR 44M (per 31/12/2024, according to the Dutch chamber of commerce). They operate worldwide. According to their own indications in a publicly available company presentation, Koks’ main markets are the Netherlands, Saudi-Arabia, Singapore, Germany, Poland, South-Africa, Kazakhstan, the United Kingdom, Russia and Australia. This means that EP074 covers a number of important markets for KOKS, and parallel patents are in force in some others.

It is unknown what share of Koks’ revenue is generated using the ultrasonic baths from this analysis.

# Recommendation

We understand it is Tech Sonic’s objective to have KOKS cease the offering, manufacture and sales of the devices rather than entering in a licensing arrangement with Koks. We further understand that Tech Sonic prefers to solve the issue without having to resort to litigation.

We recommend issuing a warning letter wherein KOKS infringement is clearly spelled out, and wherein we announce cross-border litigation against Koks as well as its group companies and distributors wherein we will seek injunctive relief, damages, recall of sold products, and surrender of KOKS’ profits. The threat of litigation and Koks’ worldwide exposure may bring them to the table to discuss Koks giving up the product in question.

- --- ooo ---

- RB, MG